GUARDS AND GUARDING

TEST PAPER (T21)

CANDIDATE NAME: ……………………………………… GROUP NO: …………….

PASS

Needs more training: 		Date: ………………………….

ASSESSOR: ……………………………….. PRINT ………………………………..SIGN

1	Select a definition under BS 5304, from those listed below for the following:

Machinery       (          )   Interlock	            (          )     Guard		         (          )

Risk		(          )   Failure to Safety      (          )     Safe Working Practice   (          )

Safeguard	(          )   Hazard	            (          )     Failure to Danger	         (          )

Safety Device  (          )   Danger		(          )     Integrity		         (          )

DEFINITIONS

(A)	A device other than a guard that eliminates or reduces danger.

(B)	A state or condition in which personal injury is reasonably foreseeable.

(C)	A safe system of work, i.e., a method of working that eliminates or reduces the risk of injury.

(D)	The ability of devices, systems and procedures to perform their function without failure or defeat.

(E)	Safety devices that interconnects a guard with the control system or the power system of the machinery.

(F)	A combination of the probability of injury and the degree of the injury.

(G)	Any failure of the machinery, its associated safeguards, control circuits or its power supply that leaves the machinery in an safe condition.

(H)	Any failure of the machinery, its associated safeguards, control circuits or its power supply that leaves the machinery in an unsafe condition.

(K)	A situation that may give rise to personal injury.

(L)	Physical barrier that prevents or reduces access to a danger point or area.

(M)	A guard or device designed to protect persons from danger.

(N)	Apparatus for producing or applying power, having fixed or moving parts each with definite functions.

2	From the drawing in Figure 1, indicate on the drawing the possible positions for the hazards identified below and describe  how they can occur.

- Cutting hazards.

- Shear hazards between two machinery parts.

- Shear hazard between machinery part and the work piece.

- Stabbing and puncture by flying objects.

- Stabbing and puncture by rapidly moving parts of machinery or pieces of material.

- Impact hazards.

- Crushing hazards.

(insert drawing)