| No | TASK TICK BOX ON COMPLETION OF EACH TASK | YES | NO |
| --- | --- | --- | --- |
| 1 | Isolate the machine |  |  |
| 2 | Lay a piece of wooden board on the bed under the spindle |  |  |
| 3 | Place a lifting rod in the chuck and clamp |  |  |
| 4 | Place the machine in neutral so that the spindle rotates easily by hand |  |  |
| 5 | Select the correct size key to release the cam-locks |  |  |
| 6 | Undo all the cam-locks by turning them anticlockwise until the location marks line up |  |  |
| 7 | Using a hide mallet gently tap the chuck free from the taper whilst holding onto the rod |  |  |
| 8 | Lower the chuck onto the wood then place the chuck in a safe and secure position. |  |  |
| 9 | Clean the spindle nose removing any swarf or debris |  |  |
| 10 | Select the - Chuck , Collet, Faceplate, Drive plate required for the task |  |  |
| 11 | Place a lifting rod into the chuck and tighten |  |  |
| 12 | Lift the chuck onto the wood using the rod |  |  |
| 13 | Clean the location faces & taper ensuring they are free from swarf & debris |  |  |
| 14 | Place the rod down the spindle and lever the chuck into position ensuring the alignment marks on the chuck and the spindle meet |  |  |
| 15 | Evenly tighten the cam-locks by turning clockwise- when fully tightened the marks must be positioned between the vee's |  |  |
| 16 | Remove the wooden plate & lifting rod and store safely |  |  |

THE PROCEDURE FOR REMOVAL &

RE-FITTING OF CENTRE LATHE CHUCKS