LESSON PLAN

LESSON PLAN No: E-LP-003	Dept: Electrical

SUBJECT: PLUGS AND CABLES

AIMS:  At the end of the session the apprentice will be able to demonstrate how to prepare a flexible cord
& terminate it to a 3-pin 230V and 110v plug using suitable hand tools in a safe manner & to a suitable standard.	Previous Knowledge: Minimal / None

| Target Time | Learning Reference | Objectives / content | Training Officer Activity | Learner Activity | Resources |  | Assessment |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Handouts | Equipment |  |
| Session 1 1:25 to 1:50 hrs | Plugs | Design / construction L/N pin insulation Reasons for use Incorrect installation – consequences & avoidance | Presentation E-CP-003 Lecture & promote observation of support materials & interactive discussion Demonstration of cable preparation & core termination Observation of hand-tool use during the assignment Visual inspection of plug & cable during the assignment | Observation & participation in group discussion Participate in the Risk Assessment process Prepare cable & terminate to plug to the required standard | Reference to E-CN-001 : Electrical Course Notes for identification of conductor colours if necessary E-PA-004 - Domestic Plug (2014) | Various examples of cables & plugs. Utilise workshop equipment & associated installations where practical Hand-tools, BS1361 plug BS4343 plug & length of suitable 3-core cord | Observation / Q&A individuals & group. Content of Risk Assessment Visual assessment of activity during the practical assignment & of the completed plug & cable assembly |
|  | Cables & Cord | Differences between cable & cord Cable / conductor sizes / current-carrying capacity Reasons for use (environment, mech. strength) |  |  |  |  |  |
|  | Hand tool safety | Correct methods of use Potential hazards Consequences & avoidance |  |  |  |  |  |
|  | Cable preparation | Removal of outer insulation Core identification Individual core length Core insulation removal methods & preparation of exposed conductor |  |  |  |  |  |
|  | Circuit protection | BS1362 Fuse / Fuse rating / Fuse carrier Visual inspection |  |  |  |  |  |

| Target Time | Learning Reference | Objectives / content | Training Officer Activity | Learner Activity | Resources |  | Assessment |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Handouts | Equipment |  |
| Session 1 (cont’d) | Cable termination | Terminals / cord grip Aligning / securing core conductors Hazards associated with terminal screwdrivers / position of plug when terminating cable | (as previous) | (as previous) | (as previous) | (as previous) | (as previous) |
|  | Personal Protective Equipment (PPE) | Potential hazards Risk Assessment Reasons for use |  |  |  |  |  |

| Equality & Diversity | Differentiated Learning Activities (Highlight or tick where applicable) |
| --- | --- |
| Challenge stereotypes by: - Using non discriminatory language. - Using diverse images, names & scenarios. Promote a positive learning environment by : - Being a positive role model. - Encouraging collaborative work. - Valuing individual characteristics and experiences. - Encouraging learners to participate. - Nurturing mutual respect. - Promoting positive behaviour. Promote inclusion by : - Anticipating and addressing potential barriers to learning. - Using accessible resources. | - Was a mix of individual, paired and group work required?  - Variety of activities for different learning styles.  - Is learning support required  (If ticked record in action plan) - Targeted differentiated questioning to check learning with prompts / leads to support less able learners.  Specific Differentiation activities: - By Extension  (For example additional practical tasks after completion of ROA/QCF/TECH CERT content) - By Group Work  (For example grouping learners of mixed ability y to support each other within the peer group) - By Content.  (For example study different resource materials within the same topic area) - By Activities.  (For example study the same content but do different activities) - By Gradation  ( For example the same information and activities are given as the activity becomes progressively more difficult) |
| LESSON NOTES |  |