LESSON PLAN

LESSON PLAN No: E-LP-018	Dept: Electrical

SUBJECT: DOMESTIC LIGHTING INSTALLATION	Previous Knowledge: Minimal / None

AIMS:  At the end of the delivery session the apprentice will have improved knowledge and understanding of the basic
principles of domestic lighting installations and be able to demonstrate how to install associated cables and
equipment and how to test circuits using appropriate test instruments and methods.

Note: -	this lesson may be taught in conjunction with the Domestic Power Installation therefore common aspects, such as circuit protection, cable preparation and installation, and installation testing, may be combined.

| Target Time | Learning Reference | Objectives / content | Training Officer Activity | Learner Activity | Resources |  | Assessment |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Handouts | Equipment |  |
| Session 1 1:25 to 1:50 hrs | Domestic installation | Review of previous session IET Regs Purpose of individual circuits & how they are protected Consumer Unit | Lecture & promote Q&A & interactive discussion as the session progresses Reference to IET Regulations (17th Edition) as required | Observation & participation in Q&A & group discussion Reference to IET Regulations (17th Edition) as directed &/or required | IET Regulations (17th Edition) On Site Gide Lighting : schematic & wiring diagrams for single way circuits | Dry marker board & pens Consumer Unit Ceiling rose Single way switch Two-way switch Intermediate switch Switch patress Cable – flat twin-&-earth (1mm2) Miniature Circuit Breaker (MCB) | Observation / Q&A individuals & group during delivery & during the group assignment |
|  | Lighting | Lighting circuit equipment (Availability, assessment of general characteristics) Maximum Demand & Diversity Cables (ratings, external influences) Circuit protection – availability type & rating |  |  |  |  |  |
|  | Single way Two-way & intermediate switched lighting circuits | Selection of equipment for lighting tasks Workplans Tools Equipment | Provide basic information to start the activity 1st 2nd fix Provide further assistance / information / guidance as the activity progresses | Observation & participation in group discussion during the assignment & during the feedback |  | Dry marker board & pens |  |

LESSON PLAN

LESSON PLAN No: E-LP-018	Dept: Electrical

SUBJECT: DOMESTIC LIGHTING INSTALLATION	Previous Knowledge: Minimal / None

| Target Time | Learning Reference | Objectives / content | Training Officer Activity | Learner Activity | Resources |  | Assessment |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Handouts | Equipment |  |
| Session 2 1.40-1.50 hrs | Inspection & testing (Instruments & methods) | Visual inspection requirements Test instruments – safe handling & operation Electrical testing – Units, values & methods (see Addendum) : - Continuity test - Insulation test | Lecture & promote Q&A & interactive discussion as the session progresses Demonstration of testing procedure / process & how to interpret results See E-LP-018 (Addendum) & reference to the IET Regulations (17th Edition) as required | Observation & participation in group discussion & feedback Reference to IET Regulations (17th Edition) & Domestic Lighting & Power Worksheet E-A-008 as directed &/or required | IET Regulations (17th Edition) – Part 6 Chapter 61 IET On-Site Guide – Sections 9 & 10 Domestic Lighting & Power Worksheet E-AG011 | Hand tools (terminal screwdriver) 500/1000V Megger & test leads | Observation / Q&A individuals & group during delivery Review of Risk Assessment content |
|  | Health & Safety | Visual inspection of hand tools, practical assignment equipment & local work environment Preparation of equipment Workplan | Promote close inspection of equipment & identify potential hazards Demonstration : preparation of cable & termination, safe use of hand tools & equipment to complete the practical assignment | Observation & participation in group discussion & feedback | Domestic Lighting & Power Worksheet E-AG011 | Hand tools & test instruments |  |
|  |  | Risk Assessment | Provide assistance / information / guidance on demand during the activity & inspect the completed Assessment | Group assignment - complete a Risk Assessment of proposed practical activity | Risk Assessment form HS-R-048 | Dry marker board & pens |  |

LESSON PLAN

LESSON PLAN No: E-LP-018	Dept: Electrical

SUBJECT: DOMESTIC LIGHTING INSTALLATION	Previous Knowledge: Minimal / None

| Target Time | Learning Reference | Objectives / content | Training Officer Activity | Learner Activity | Resources |  | Assessment |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Handouts | Equipment |  |
| Sessions 3 & 4 3.40-7.5 hrs | Lighting installation Test & Inspection (practical methods) | Using the notes in E-AG011 & the previously completed schematic diagrams, construct Lighting Installations for : single-way switching two-way switching intermediate switching Visually inspect & test each circuit using approved methods & instruments (see E-LP-017 Addendum) Record & interpret results | Provide information / guidance / support on demand during the assignment activity Demonstrate Inspection & Testing methods on completed installation Observe individuals & conduct Q&A during the Inspection & Testing activity Highlight hazards prior to connection of completed circuits to the supply Connect, visually test &, when complete, remove the mains connection. | Practical assignment - Apprentices to work in teams of 2 & follow the instructions in the Worksheet (E-AG011). Demonstrate installation methods. Observe demonstration of Inspection & Testing methods Demonstrate how to inspect & test completed circuits (to be observed by the Instructor). Interpret & record inspection & test results in the relevant section of the Worksheet (E-AG011) Observe final testing by Instructor | Domestic Lighting & Power Worksheet E-A-008 | Work board (5’x4’) Consumer Unit with 6A MCB Ceiling rose (x2) Single way switch Two-way switch (x2) Intermediate switch Switch patress (x4) Cable – flat twin-&-earth (1mm2) Cable clips Hand tools Earth sleeving Live sleeving Test lead & RCD (instructor use ONLY) | Observation of practical activity & assessment of knowledge during practical assignment Note : connection to the mains supply to assess the completed circuits may be repeated if a fault is observed / detected |
| Sessions 4 (cont’d) | Lighting installation | Complete a written assignment of the practical task as instructed in the Worksheet E-AG011 | Provide information / guidance / support on demand during the assignment activity | Complete written assignment as instructed in the Worksheet (E-AG011). | Domestic Lighting & Power Worksheet E-AG011 | Practical task notes Pen, pencil, ruler, lined paper | Assessment of written content of assignment Note : content may be aligned with the Domestic Power assignment hence common aspects need not be repeated |

LESSON PLAN

LESSON PLAN No: E-LP-018 (ADDENDUM)	Dept: Electrical

SUBJECT: DOMESTIC LIGHTING INSTALLATION (TEST PROCEDURE)	Previous Knowledge: Minimal / None

Prior to making the final connection to the mains (230Vac) supply:

Inspection

Circuits must be visually inspected as per the Worksheet (E-AG011)

Continuity Testing

- At the Consumer Unit, ensure all lighting circuit conductors are disconnected and accessible for visual inspection and testing

- Prepare and/or calibrate the Test Instrument, select Ohms (Ω) and attach the leads together in order to assess the test-lead resistance

- At the final Ceiling Rose, remove the cover, disconnect the lamp holder and cross-connect the Phase (Loop) and CPC terminals

- At the Consumer Unit, connect the test leads to the same cores and observe the test reading (R1+R2). Record the continuity resistance value in the Worksheet table

- At the final Ceiling Rose, cross-connect the Phase (Loop) and Neutral terminals

- At the Consumer Unit, connect the test leads to the same cores and observe the test reading (R1+Rn). Record the resistance value in the Worksheet table

- At the final Ceiling Rose, disconnect and remove the cross-connection, reconnect the lamp holder to the appropriate terminals and refit the cover

Insulation Testing

- Check all circuit equipment is connected and ensure any lamps have been removed

- Select “500V MegOhms” (MΩ) on the Test Instrument

- At the Consumer Unit attach the Test Instrument leads to any two of the lighting circuit conductors (CPC-Live, Live-Neutral, CPC-Neutral)

- Press the “Test” push-button on the Test Instrument and keep it pressed until a steady reading is obtained

- Observe and record the value of insulation resistance in the Worksheet table

- At the Consumer Unit, repeat the test across the remaining two pairs of conductors and record the insulation values accordingly

On completion of the testing process, reconnect the conductors to their relevant terminals in the Consumer Unit

| Equality & Diversity | Differentiated Learning Activities (Highlight or tick where applicable) |
| --- | --- |
| Challenge stereotypes by: - Using non discriminatory language. - Using diverse images, names & scenarios. Promote a positive learning environment by : - Being a positive role model. - Encouraging collaborative work. - Valuing individual characteristics and experiences. - Encouraging learners to participate. - Nurturing mutual respect. - Promoting positive behaviour. Promote inclusion by : - Anticipating and addressing potential barriers to learning. - Using accessible resources. | - Was a mix of individual, paired and group work required?  - Variety of activities for different learning styles.  - Is learning support required  (If ticked record in action plan) - Targeted differentiated questioning to check learning with prompts / leads to support less able learners.  Specific Differentiation activities: - By Extension  (For example additional practical tasks after completion of ROA/QCF/TECH CERT content) - By Group Work  (For example grouping learners of mixed ability y to support each other within the peer group) - By Content.  (For example study different resource materials within the same topic area) - By Activities.  (For example study the same content but do different activities) - By Gradation  ( For example the same information and activities are given as the activity becomes progressively more difficult) |
| LESSON NOTES |  |