# Safety Operating Procedures

# Hand Tools

DO NOT use this machine unless a Training Officer has instructed

You and has given permission by a signed Risk Assessment

| Safety glasses must be worn at all times in work areas. | Long and loose hair must be contained. | Bench Gloves must be worn when using hand tools |
| --- | --- | --- |
| Sturdy footwear must be worn at all times in work areas. | Close fitting/protective clothing must be worn. | Rings and jewellery must not be worn. |

## PRE-OPERATIONAL SAFETY CHECKS

- Ensure no slip/trip hazards are present in workspaces and walkways.

- Ensure that all tools are in good condition before use.

- Specifically check File handles, Hacksaw blades, Hammers loose heads –damaged handles. Scribers & Punches are sharp.

- Check the bench vice is secured and in good condition

- Always use soft jaws use in the vice to avoid damage to the work-piece.

- Visually check power tools for damage - condition/cables/PAT test.

- Select the correct tool for the job.

- Faulty equipment must not be used. Immediately report.

## OPERATIONAL SAFETY CHECKS

- Clamp work piece firmly into the vice. Ensure minimum overhang.

- Hold hacksaws with both hands, cut on the side of the vice which is your dominant hand, slow at the end of the cut.

- When using files use good balance

- When marking out scribe away from yourself

- When using hammers hold away from the head and avoid being in the line of fire.

## HOUSEKEEPING

- Return all tools & equipment to their respective storage areas.

- Safely store precision measuring equipment.

- Clean up and absorb any spills immediately.

- Quarantine any faulty or damaged tools/equipment.

- Leave the work area in a safe, clean and tidy state.

- Safely dispose of any waste.

## POTENTIAL HAZARDS AND INJURIES

- Sharp Tools– contact injury handling /clean-up.

- Moving parts - hair/clothing entanglement.

- Eye injuries.

- Skin irritation.

- Metal splinters/swarf	burrs.

- Sharp edges and burrs.

- Burns

- Noise (use protection when required)

- Manual Handling -heavy /long sections of material

## DON’T

- Hacksaw with one hand.

- Leave power tools running unattended.

- Attempt to hold or catch the work-piece as it completes the cut.

- Carry sharp tools in pockets.

- Get in the line of fire.

- Horseplay